The ZING MkIV CLG

An Easy To Build Fun Type Glider

The Zing series of CLG models was an attempt to build a rather

simple glider that could provide enough performance to keep the

builder's interest in continued flying even after the first few flights.

We tried for a glider that could be launched in an almost vertical climb and at the top would roll out into a nice flat glide and even begin to hunt for that elusive thermal activity. It appears that we have come very close to achieving the near vertical climb and our prototype rolls out rather nicely at the top and the glide is pretty flat. The prototype has been turning in flights consistently in the 30~35 second range in calm air.

The fuselage is made up of two pieces of 1/8” x 1/4” spruce. The main fuselage is 18” long with a taper from the trailing edge of the wing. Taper down to 1/8” and be sure to leave that last little 1/4”  at the end. We put a small piece of 320 sand paper on each side to give a good grip for the launch.

There is a second piece of 1/8” x 1/4” spruce that is 5” long.This is used on the top front of the fuselage and provides a “cap” over the leading edge of the wing.

The wing panels are prepared from 3/32” sheet balsa and given an airfoil shape as shown in the fuselage view. Here we used 3/32” balsa from the scrap box and didn't try to get the best and lightest piece possible, we used what was on hand. I am sure you can do much better here and possibly end up wit a much lighter model than we did. Use the amount of dihedral shown, it is probably close to 12 degrees for each wing panel.

For the horizontal and vertical tail pieces we used 1/32” balsa that came from the scrap box. Once again if you choose the balsa wisely you probably can do much better than we did.

In mounting the horizontal tail to the fuselage we used a small amount of “stab tilt” to get a nice left turn in glide. See the “stab tilt” sketch to see how we did it. Just make sure that the piece of material you use to get the “stab tilt” is as long as the chord of the horizontal tail. Do this so that the horizontal tail is still at 0 degrees of incidence.

When the vertical tail is glued to the fuselage make sure that it's trailing edge is directly above the trailing of the horizontal tail.

After the wing is glued to the fuselage the top front piece of spruce can be glued on. This provides a “cap” over the wing leading adge and provides some strength in this area.

Remember we want a model with 0/0 incidence.

Add your catapult launch hook.

I leave the finish on the model up to you, most folks have their own method so use the one that works for you.

Note that we do not show a DT installation but suspect that a good type would be the front mounted beer can aluminum flap type with the fuse mounted on the front top of the fuselage.

This glider should have a very good glide ratio, a very flat glide when you make your first check. Adjust the amount of ballast to get it right.

Our protoype weighed in at 15.3 grams.

We used the standard catapult launcher, a 6” dowel and a 9” loop of 1/4” super sport.

Start your catapult test with launches of 70~80 degrees rather than vertical and maybe  75% of the catapult pull. If it looks good....go for it.

If you have questions or comments e-mail us at ltcdtret@swbell.net. Sure would like to hear about you successes with this model.